UV Lamps

This range of low pressure Mercury-Arc lamps is designed to emit light radiation peaking in the ultra violet region at 253.7nm (UV-C). The ultra violet light output at this wavelength is typically used for erasing Eproms. Prolonged exposure may also be effective in passivating micro-organisms such as bacteria, viruses, yeast and mould.

| Lamp Code | Dimensions |  | Watts (W) | Current (A) | Voltage (V) | Spectral Peak (nm) | UV Output (W) | Ave Life (h) | Base |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Length (A) (mm) | Diameter (B) (mm) |  |  |  |  |  |  |  |
| G4T5 | 134.5 | 15.5 | 4.6 | 0.162 | 30.0 | 253.7 | 0.8 | 6000 | G5 |
| G8T5 | 287.0 | 15.5 | 7.9 | 0.170 | 56.0 | 253.7 | 2.5 | 6000 | G5 |
| G15T8 | 436.0 | 25.5 | 14.7 | 0.300 | 55.0 | 253.7 | 4.8 | 8000 | G13 |